Revamp Your Tank: Frequently asked questions

The scheme has now been rolled out, with several additions and amendments made to the FAQs as further details have emerged. The scheme is being delivered by Wiltshire Council in partnership with the Wessex Rivers Trust.

Please note that the scheme is directly link with ‘Nutrient Neutrality’ and, as such, is constantly changing to reflect new national guidance from Natural England or legislative changes from central government. There are new changes expected in 2026, off the back of the Planning and Infrastructure Act achieving Royal Assent in Dec 2025, that may affect the mechanisms of delivery of Revamp Your Tank. However, it is the intention that the scheme will still run, subject to market demand for credits (more information below).

- What does the scheme cover and how does it work?

Revamp Your Tank cannot be used to retrospectively fund projects that have already been agreed or completed. The scheme is funded through central government pump priming to improve water quality in our rare chalk streams. It is also helping unlock new development through nutrient mitigation, as part of nutrient neutrality. Nutrient neutrality is a legal requirement that ensures any additional nutrient load from new development is fully offset; upgrading septic tanks is one of the mitigation measures the council is delivering. Progression of projects is directly dependent on the sale of nutrient credits-priming to improve water quality in our rare chalk streams and to help unlock new development through nutrient mitigation as part of nutrient neutrality. Nutrient neutrality is a legal requirement that ensures any additional nutrient load from new development is fully offset; upgrading septic tanks is one of the mitigation measures the council is delivering. Progression of projects is directly dependent on the sale of nutrient credits

The scheme will cover the full provision and installation of a new package treatment plant (the New Plant) to replace your old system (subject to the scheme’s conditions and cost cap).

This will include:

- An initial survey carried out by a contractor and an assessment of your current system’s performance.

- The contractor will design and procure the most appropriate New Plant for your property subject to the terms of the scheme.

- Groundworks associated with the installation of the New Plant.

- Removal and/or decommissioning of the old system.

- Installation of the New Plant and any required services, including electrical supply.

- Making good of the ground and leaving the area as found, as much as is possible (this may involve spreading grass seed over the newly disturbed soil).

- The first annual ‘check-up’ service to ensure proper function and maintenance. This will include a tank empty, but not include any operational costs i.e. running costs

- Who is eligible?

The pre-requisites to being eligible to apply for this scheme are:

- The property must be within the Hampshire Avon catchment and in Wiltshire

- The applicant must be the homeowner or have the homeowner's permission

- The project cannot already be agreed and/or completed.

- The property must be residential, not a business.

After this, the council carry out several screening stages to assess project locations against Natural England Small Scale Thresholds, the council’s Local Development Order criteria and hydrological connectivity. Projects are then categorised and prioritised or rejected, based on the outcome of the screening.

Please note that the council uses the most accurate and detailed mapping tools at its disposal to inform the process. Also, the scheme is dependent on the demand from developers for nutrient credits in your location. These credits arise from the improved performance of your new tank compared to the old one. Therefore, the fact that your neighbour’s tank is being installed is not a guarantee that your project will go ahead.

- Is the scheme run on a ‘first come first serve basis’ and which areas are you prioritising?

No the scheme is demand led and is not on a first come first serve basis.

We prioritise the headwaters of the Avon’s tributaries to meet Natural England’s requirement that nutrient mitigation is delivered upstream of the development it offsets. We also need to focus delivery on sub-catchments-catchments where the resulting nutrient credits are needed to mitigate development so that means project progress align with market demand, which has slowed considerably.

The initial wave of installations in 2025 cleared the existing development backlog, and the less complex projects covered by the Local Development Order were prioritised. Having successfully delivered installations—particularly in the headwaters of the Wylye and Nadder—we are now (updated Feb 26) giving priority to applications from residents living close to rivers or in flood-prone areas within the following catchments:

• Upper Till (Shrewton and upstream, Tilshead, Maddington)

• Upper Bourne (Porton, Allington, upstream of Cholderton, Collingbourne Ducis near the river)

• Headwaters of the Hampshire Avon (Burbage, Easton Royal, Coate, Etchilhampton, Urchfont, Wedhampton, All Cannings, Stanton St Bernard, Honeystreet, Woodborough, Patney, Wilsford, Marden, Beechingstoke, Manningfords, Pewsey, Southcott, Milton Lilbourne)

Whilst the scheme is expected to continue for the foreseeable future, with funds from nutrient credit sales reinvested into further projects, anticipated updates from Natural England may require adjustments to the way it is delivered.

- How will the funds be dealt with and when will they be paid?

The contractor will be paid directly by the Council and will not ask for any money or bank details from you, as the customer, unless any variations to the design and installation plans have been made by you and privately quoted to you, or if the project has extra complexities/costs over and above the fund threshold (cost cap). If the latter is true, the extra costs and works associated will be explained to you by the contractor, and you will decide whether to progress or not.

- What is the process from application to delivery?

Your application is only an expression of interest in the scheme, and an acceptance puts you in the queue for consideration. The progression of your project is directly linked to the demand and supply of mitigation credits in your area (see Q1) and will be dictated by the criteria set out in Qs 2 & 15. The project is also only certain when you have signed a contract directly with the contractor and, until such time, the council reserves the right to remove the project from the scheme.

- Do I have to organise anything?

If your sewage system only serves your property, you will only need to coordinate with our third-party contractors once the project starts and arrange site visits, dates and access for works etc. All other project management and payment will be cared for directly between the council and the contractor.

If there are multiple properties involved, neighbours will need to coordinate acceptance of the scheme and, if in agreement, appoint a ‘lead applicant’ for ease of communication. This is done through signing extra documents.

- Why are septic tanks thought to pollute rivers?

They are designed so that the liquid and solid waste is separated, with the liquid being released into the surrounding soil (a drainage field) where it can be assimilated (‘eaten’) by the bacteria naturally occurring in the soil. However, sometimes septic tanks have been placed too close to surface water bodies, or they become affected by groundwater or, increasingly, surface water flooding. In this way, the effluent they release can reach rivers and streams as pollution. Moreover, if a septic tank malfunctions, it may also release untreated effluent.

- What is better about the new system?

Package treatment plants (PTPs) not only separate the waste, they also biologically treat them to produce bacteriologically safer and clearer water. This is why they are legally allowed to discharge straight into a water course, whereas septic tanks are not. The PTPs being used in this scheme are also designed to remove some excess nutrients from the wastewater, which cause pollution if released. In addition, if tertiary treatment is used in series with a PTP, this results in the highest water quality standards being reached.

- What’s in it for me?

This is a scheme designed to primarily benefit the environment. However, as well as having the substantial cost of replacing your system covered and having the project managed on your behalf, the funding will also cover a 1 year service plan and one tank empty. These benefits are, of course, additional to helping to reduce pollution inputs into our waterways. This scheme also can be a benefit in the case of buying or selling houses (see Q27).

- How long will the new system last?

Roughly 30-40 years if it is well maintained. Maintenance schedules and model specific information will all be given to you by the contractor on completion of the project.

- Is the new package treatment plant likely to smell?

If your system is well maintained and serviced according to the manufacturer’s guidelines, it should not smell. Foul odours will be a symptom of an issue.

- Is the new package treatment plant likely to make a sound?

Yes. A sewage plant requires the addition of oxygen into the water which is added through an air blower or air compressor. This can produce a humming sound. Our specification of the systems to be considered for use dictate they must run below 50dB.

- Are the proposed PTPs compliant with the current legislation?

Yes, they will be compliant with all environmental permitting regulations such as the Environment Agency’s General Binding Rules

- What are the timeframes?

Projects continue to be progressed. However, the significant initial delivery will now slow to fall in line with market demand for credits. Projects will be prioritised according to a) the simplicity of the works and permissions to be sought b) the geographical location and neighbouring project sites where possible (see Q15) c) connectivity to the river either via surface or groundwater (New Guidance awaited).

Projects such as having multiple properties connected to a shared tank, or having the possibility of connecting to Wessex Water’s mains network will take more time and are legally more complex.

There is currently a significant queue of PTP installations and limited demand for nutrient credits from developers in some areas. The scheme must not be treated as guaranteed support. If your septic tank is failing or legally non-compliant, you are strongly advised to pursue a private solution rather than waiting for potential project delivery.

- Will the scheme fund upgrades to recently installed housing association owned sewage treatment units? Including newer ones?

We are targeting the older septic tanks first, so would not be looking to replace recently installed PTPs.

- My neighbour’s tank went in quickly but I’ve been waiting a long time – why?

It is possible that there could be a significant time delay between the two applications in a similar area. The first batches of prioritised projects were sent to the contractors to clear the backlog of demand from developers at the start of the scheme. Since then, demand for credits slowed with projects now being progressed at a slower pace and is developer demand led.

Furthermore, your property may not meet the same criteria as a neighbour’s. Your location/property may present more complexities than your neighbours’ which could require a planning application or more permissions.

- How long will the average planning application process be for septic tank instalment?

Where we can, we use a more streamlined process for planning in the form of a Local Development Order (LDO). This provides a blanket planning permission for projects of this nature that meet the Order’s criteria. Some projects will meet the criteria of the LDO and, therefore, be prioritised. Others, such as multiple properties sharing a single tank, or where a listed building is involved, will still require planning permissions and will take longer. All processes are dictated by the demand for credits in the area by developers (see Q1).

- Are there any other schemes of this nature already delivered in the locality?

Yes, another privately led scheme has been successfully delivered in Somerset and is now expanding into Wiltshire.

- What’s included in the 1 year funded service plan?

An annual check-up and service is included with one tank empty. Operational costs such as electrical charges are not included.

- We have multiple properties connected to a single septic tank. Are the Package Treatment Plants (PTPs) on offer large enough to account for such scenarios?

Yes, the contractor would size the PTP accordingly to demand from the properties. Please note, these ‘multiple property’ projects are not covered by the local development order mentioned in Q17. As such, they will take a bit longer to complete and require more coordination from residents.

- Will the permit to discharge to the river be sought on the homeowner’s behalf?

Yes, the contractor and the council will be ensuring all planning, building regulations and environmental permitting requirements are met.

- If there is an option for my property to be connected to mains sewerage, can Wiltshire Council offer any financial contribution to the process?

We are currently talking to Wessex Water about connecting properties to the mains sewer, where properties are believed to be close to the network and connection is without undue complication. Connecting to the mains, if an option, could be covered by the scheme and will be taken forward on a case-by-case basis. If your property is found not to be possible to connect to the mains, you could still have a PTP installed under the scheme.

- We have an existing maintenance contract in place for the emptying of sludge from our septic tank. Is the emptying process for the PTPs the same as septic tanks? I.e. will our contract have to be amended?

Your contractor who empties your tank will, most likely, be familiar with the functioning of the PTP and, therefore, know how to empty it. Any contract amendments will entirely depend on your particular contract wording.

- How will I know if my project is being progressed and can I expect to hear from a contractor?

We are keeping everyone up to date with the status of projects as much as possible via the website and where there is a change in your status via from the Revamp Your Tank address. If you don’t hear anything for a while, it means there is no update from the last position. When your project is forwarded to a contractor, Wiltshire Council will contact you to confirm this, and that you can expect to hear from the contractor. Rest assured that if something changes, you will be notified.

As this scheme is market demand led and is reliant on the needs of developers the council is not able to control the speed or locations of projects. There is no guarantee that your tank will be upgraded until you have a signed contract.

- Will some Package Treatment Plants discharge to ground? Or do all have to discharge to a watercourse? Will the proximity to a watercourse impact eligibility?

The system can discharge either to ground or into the water course. The first eligibility criterion is being situated in the Hampshire Avon catchment. If you are in the headwaters of a tributary of the Avon (as mentioned in Q3) and a single house connected to your tank, then your project will be the highest priority. All other eligibility is determined through multiple screening processes using multiple GIS mapping layers, therefore it cannot be strictly defined before the application stage.

- Can I, as the customer chose the package treatment plant and contractor?

Due to the scale of the project, complexity involved in public sector procurement processes, and the need for us to prioritise and group projects for efficiency, the council will choose the contractor for your project. The package treatment plant will also be decided by the contractor and the council.

However, you, as the customer, will have the ultimate sign off on all works once the contractor has explained the survey results and recommendations to you. You will then enter into a legal agreement with the contractor at that stage, should you accept the information presented to you.

- If my current septic tank operates to the required standard, do I still legally need to upgrade?

You will need to establish if your plant drains to surface water (General binding rules: small sewage discharge to a surface water - GOV.UK (www.gov.uk)) or to ground (General binding rules: small sewage discharge to the ground - GOV.UK (www.gov.uk)) and work through the points to establish your answer. However, pay close attention to Rule 6, copied in below, which dictates you MUST upgrade if your plant discharges to surface water:

### “Rule 6: use the correct treatment system

You must use a small sewage treatment plant to treat the sewage if you’re discharging to a watercourse such as a river or stream. A sewage treatment plant, also known as a package treatment plant, treats sewage to a higher standard than a septic tank.

If your septic tank discharges directly to a watercourse, as soon as possible you must do one of the following:

- connect to a public foul sewer

- replace your septic tank with a small sewage treatment plant

- install a drainage field (also known as an infiltration system) – a series of pipes with holes placed in trenches and arranged so that the waste water can trickle through the ground for further treatment – and check if you meet the general binding rules for discharges to ground

You must have plans in place to do this work within a reasonable timescale, usually 12 months.”

#### If you buy or sell a property with a septic tank that discharges directly into a watercourse

You should agree with the buyer or seller who will be responsible for replacing or upgrading the treatment system. You should agree this as a condition of sale.

- Are the contractors carrying out the work going to be liable for any damage caused to the property/infrastructure?

If you decide to go ahead after the site visit, you will enter into a contract with the contractor which will set out your rights and the duties the contractor owes you. This will explain to what extent the contractor will be liable for any damage caused. This will also set out how the contractor will repair or reinstate the ground after installing the new system.

Please note that the contractor may not be able to repair or reinstate the ground exactly as it was before the new system was installed. You must read this contract carefully and decide if it is suitable for you.

- If the site visit deems the property needs an upgrade, but turns out it’s not eligible for some reason or another, will I be reported as being non-compliant?

You will not be reported for non-compliance of your tank. However, we have a responsibility to contact relevant agencies if we become aware of any behaviours or actions, besides the need for a tank upgrade, that are potentially harmful or dangerous to the public or environment.

If your tank is non-compliant, we would strongly advise you to upgrade your plant via another method as soon as possible, if it is not eligible for the Revamp scheme.

- What is typically required in terms of maintenance of the PTP, and what is the estimated annual cost?

Depending on the make and model recommended for your property, the average maintenance includes an annual empty and a service. The costs of these will be explained to you after your site visit, as they are dependent on the model recommended by the contractor, and you will have ultimate sign off on the project. There may be other providers that could provide these services but please do check with your contractor first.

- Natural England state nutrient mitigation must be delivered in perpetuity for 80-100 years. What impact will the upgrade have on the deeds to my property?

Once the plant is in place, and the 1st year of service and maintenance has been honoured as part of the scheme, the plant is entirely yours to run and look after. The plant will be covered by the Local Development Order (mentioned in Q17) or a separate planning permission. This requires that future replacements of the system (for example in 30 to 40 years) shall have an equivalent treatment efficiency of at least 5 mg/l for Total Phosphorus removal and must meet requirements of relevant planning and environmental legislation. This includes the requirement to look after and maintain the system in line with manufacturers recommendations.

- Is there a contractual/legal obligation on the homeowner to maintain the new PTP?

Yes. After the survey stage, you will be able to choose if you proceed with the project or not. If you do, you will sign a contractual agreement with the contractor directly which will require the homeowner to operate and maintain the new system as per manufacturer’s guidelines. It is at this point when your project is considered ‘certain’.

- Is the addition of tertiary treatment technology an option with this upgrade scheme?

Tertiary systems and treatments are not funded by this scheme. However, if you would like to add this on, you would be able to have a conversation with the contractor directly and receive a quote which would be payable by you, the customer, as part of the overall works. This would be seen as a variation which, in the detailed information you will be sent if accepted onto the scheme, is outlined as being chargeable to you as an additional cost.

- If the PTP oxygenation pump stops work, how will I know?

This will be dependent on the model recommended to you by the contractor after the survey stage and, therefore, a question for the contractor.

- What happens if there are multiple properties linked to one sewage plant?

Multiple property projects are more complex and time consuming than other projects due to the coordination needed, easements and rights of access etc. Therefore, these projects are currently lower on the priority list. However, if your property is linked with multiple others, it is only required that one property makes the initial application and replies ‘multiple properties’ to the question “how many properties does this tank serve?”.  From there, the project will be screened for acceptance or rejection by the council.

If accepted, the initial applicant will receive an email with detailed information to read, as well as a multiple property project form. All neighbours must read and agree to the detailed information and send a completed form for their property in response to the acceptance email. This requires group coordination and complete buy in from all connected parties for these types of projects to progress. Please note, if you all share one tank and want to have your own individual tanks, this is not a given that this will be possible, and each project will be treated on a case-by-case basis.

- What is an Environmental Delivery Plan (EDP)

EDPs will be introduced by Natural England in a bid to streamline planning processes, approaching nutrient pollution and nature restoration in a more strategic way. The EDP effectively is a different way of delivery a mitigation scheme which will give developers a clear and simple option to discharge their obligations to avoid harming protected sites and species via the use of the Nature Restoration Fund.

These changes have come about through the Planning and Infrastructure Act achieving Royal Assent on 18th December 2025.

Procurement of installers

Wiltshire Council appointed Libra Utility Services Ltd and Willow Pumps Ltd through a competitive process under the National Framework Partnership, enabling timely procurement in line with the Public Procurement Regulations 2015. These contractors are delivering projects in line with the council’s agreed prioritisation. The Local Nutrient Mitigation Fund, awarded in December 2023, must be fully committed by March 2025, creating a very challenging delivery timeframe with limited resources. Given this pressured programme, the council has appointed two experienced civil and drainage engineering companies with the capacity to deliver the scheme and will not undertake further competitions. Although the contractors are not Wiltshire-based, all funding is being invested within the county, supporting environmental and social benefits for Wiltshire residents. While procurement is no longer required for the council-led ‘Revamp Your Tank’ scheme, developers remain free to select any scheme; sewage system installers are therefore encouraged to offer alternative, bespoke solutions to help protect water quality in the Hampshire Avon Catchment and improve the health of local aquatic ecosystems.

Procurement of installers

Wiltshire Council appointed Libra Utility Services Ltd and Willow Pumps Ltd through a competitive process under the National Framework Partnership, enabling timely procurement in line with the Public Procurement Regulations 2015. These contractors are delivering projects in line with the council’s agreed prioritisation. The Local Nutrient Mitigation Fund, awarded in December 2023, must be fully committed by March 2025, creating a very challenging delivery timeframe with limited resources. Given this pressured programme, the council has appointed two experienced civil and drainage engineering companies with the capacity to deliver the scheme and will not undertake further competitions. Although the contractors are not Wiltshire-based, all funding is being invested within the county, supporting environmental and social benefits for Wiltshire residents. While procurement is no longer required for the council-led ‘Revamp Your Tank’ scheme, developers remain free to select any scheme; sewage system installers are therefore encouraged to offer alternative, bespoke solutions to help protect water quality in the Hampshire Avon Catchment and improve the health of local aquatic ecosystems.